The SMORPHI² TransfoBot

A transforming a robot was made in collaboration with Wefaa Robotics and Singapore University of Technology and Design, for showcasing the blooming environments in the field of advance locomotion and the vast spectrum of Internet Of Things [ IoT & Automation ].

Our Prestigious Mentors:

- Dr. Gyanaranjan Panigrahi

- Mr. Pritam Nanda

- Mr. Biswajeeban Mishra

The Mighty Team:

- Sahil Kumar Chaudhury

- Tejaswinee Nayak

- Tanushree Lenka

- Choudhary Abisant Jena

- Rudra Narayan Behera

- Lokesh Patra

- Pallav Palai

“ Every step and procedure was performed in alignment with the guidance of all the mentors and the esteemed panel from Wefaa Robotics, Singapore ”

Step#1: Unpacking the smorphi² box, all the components were taken out of the box and spread on the floor, and the components were:

- 16 x Mecanum Wheels ( Right + Left )

- 4 x Aluminum Base Plates

- 8 x Acrylic Base Plates

- 16 x Mecanum Motors w/ Mounts

- 6 x Solenoids w/ Latch Mount, Guide & Catch

- 6 x Hinge Mounts

- 3 x Hinge Mechanisms

- 1 x LiON Battery

- 2 x Battery Bracket

- 1 x Masterboard [ Espressif32 ]

- 4 x Slaveboard

- Multiple Sensors [ Sound, Temperature, InfraRed ] + HuskyCamera

- Multiple Size Screws [ 3 * 5 ( x200 ) + 3 * 10 ( x45 ) + 3 * 25 ( x50 ) + 4 * 8 ( x15 ), 45 * M3 Nuts ]

- 32 x Hex MF M3 Nylon 45mm

- 28 x Hex FF M3 Nylon 10mm

- 8 x 4-pin Connectors + 4 x 8-pin Connectors

- 1 x USB Type C + 1 x Battery Charger

- 2 x Wing Screws [ M3 * 5 ] + 1 x Ceramic Screwdriver + 1 Hex Key 1.5mm

Step#2: Through the video conferencing session, we were given a link:

[ ‘ https://smorphi-documentation.readthedocs.io/en/latest/ ’ ]

- In that documentation website, there was a Smorphi Assembly Manual, of different versions, as w/o and w/ Voltage Regulator with multiple purchase periods as from:

[ May ’23 – September ’23 – March ’24 – August ’24 – P R E S E N T ]

- There, we chose the 3rd one, i.e. Smorphi² w/o Voltage Regulator [ May ’23 ~ August ‘24 ], and downloaded that manual.

Step#3: Motor Sub-Assembly [ A1 ; Pg. 7 ]

- Left & Right Mecanum Wheels are attached with their respective DC Motors, using Motor mounts, shaft sleeve and screws [ M3 * 22 ].

Step#4: Base Module Assembly [ A2 ; Pg. 8 ]

- On an Aluminum Base Plate, there are 4 grooves numbered [ 1, 2, 3, 4 ].

- Inside those grooves, the wheels are inserted according to odd-even layout as Right Wheels are Even Grooves [ 2 & 4 ], and Left Wheels are Odd Grooves [ 1 & 3 ].

- After fitting the wheels, Skirt Panels [ A & B ] are placed on the sides.

- Then, 4 x Hex M-F M3 Nylon 45mm Screws were tightened on the 4 corners of that Al Base Plate.

Step#5: Solenoid Latch Sub-Assembly [ A3 ; Pg. 11 ]

- On both sides of the Solenoid Latch, a latch mount and latch guide were attached with [ M3 * 5 ] screws, similarly 6 latches were joined.

Step#6: Mechanical Sub-Assembly [ A4-7 ; Pg. 12-19 ]

- After assembling the parts respectively, now we got to assemble each module separately, according to the manual, in a very specific way of assembly of the 4 modules.

- Module 1:

  - 2 Solenoid Latches are fitted, between 1 – 2, and 4 – 3.

  - 1 Solenoid Catch is fitted between 1 – 4

  - 1 Hinge Mount is fitted near 4th Corner of the Al Plate.

- Module 2:

  - 1 Latch is fitted between 1 — 4.

  - 2 Catches are fitted between 1 – 2, and 4 – 3.

  - 2 Mounts are fitted near 2 corners, 1 & 4.

  - Battery Brackets are installed, in the middled, tightened with a Wing Screw, along w/ the battery.

- Module 3:

  - 2 Latches are fitted between 1 – 2, and 4 – 3.

  - 2 Catches are fitted between 1 – 4 and 2 – 3.

  - 2 Mounts are fitted near diagonal /odd corners, 1 & 3.

- Module 4:

  - 1 Latch is fitted between 1 – 4.

  - 1 Catch is fitted between 4 – 3.

  - 1 Mount is fitted near the 4th Corner.

Step#7: Full Mechanical Assembly [ A8 ; Pg. 20 ]

- All 4 modules are aligned together as [ 1 -M1- 4 ] > [ 1 -M2- 4 ] > [ 3 -M3- 2 ] > [ 3 -M4- 2 ], i.e. M3 & M4 are rotated 180° from M1 & M2.

- After this alignment, the 4 modules are locked to each other, and the 3 hinges were attached to the hinge mounts.

Step#8: E-Tray Sub-Assembly [ B1 ; Pg. 24 ]

- Hex F-F M3 10mm holes were attached to the acrylic base plates with [ M3 x 5 ] screws.

- Then, the slaveboards were attached onto those holes using more screws.

Step#9: E-Tray onto Mechanical Assembly [ B2 ; Pg. 25 ]

- The Motor Connectors on the Slaveboards are oriented with the Al base plate numbers, i.e. Motor 1 > 1, Motor 2 > 2, etc.

Step#10: Motor Cable Connections [ B3 ; Pg. 29 ]

- All DC Motors of the tires are connected to their respective Motor Connectors [ M1 > MC1, etc. ]

Step#11: Solenoid Cable Connection [ B4-7 ; Pg. 30 ]

- Module 1:

  - Latch [ 1 – 2 ] connected to Solenoid 1 Connector.

  - Latch [ 4 – 3 ] connected to Solenoid 2 Connector.

- Module 2:

  - Latch [ 1 – 4 ] connected to Solenoid 1 Connector.

- Module 3:

  - Latch [ 1 – 2 ] connected to Solenoid 2 Connector.

  - Latch [ 4 – 3 ] connected to Solenoid 1 Connector.

- Module 4:

  - Latch [ 1 – 4 ] connected to Solenoid 1 Connector.

Step#12: InterModule Cable Connection [ B8 ; Pg. 34 ]

- 3 * 8-Pin Wires are connected from Head to Tail Connectors of all the boards, as

  - M1 – M2, & M2 – M3 are wired via 1 – 4 sides.

  - M3 – M4 is wired via inverse 1 – 4 side [ 180° ]

Step#13: Address Selection [ B9 ; Pg. 35 ]

- The Ceramic Screwdriver was used to adjust the rotary switch for respective address of each module as:

  - Module 1: Address 0

  - Module 2: Address 1

  - Module 3: Address 2

  - Module 4: Address 3

Step#14: Masterboard E-Tray Sub-Assembly [ B10 ; Pg. 36 ]

- Exactly as the Slaveboard attachment, the Masterboard is attached to an acrylic base plate, on Module 2.

Step#15: Masterboard E-Tray onto Main Assembly [ B11 ; Pg. 37 ]

- The header to slaveboard connector on the Masterboard is aligned to face the 1 – 4 side of the slaveboard.

- Post-alignment, the an 8-Pin wire is connected from [ Header to Masterboard ] to [ Header to Slaveboard ] connectors in both the boards, respectively.

Step#16: Acrylic Covers [ B12 ; Pg. 39 ]

- Rest of the modules [ M1, M3, & M4 ] are attached with Acrylic Covers, using M3 x 5 screws.

Step#17: Battery Connection [ B13 ; Pg. 40 ]

- Ensuring the battery is fully charged, it was already placed in the Battery Bracket, and now its wire is connected to the Battery Connector on the Mainboard.

- Finally, the toggle switch is turned on, and if all the LEDs in all modules lit up, it was assured that all connections were properly made!

- After that, the ENABLE button is held for 1second to activate the robot.

_ smorphi² Assembly Complete _

Step#18: Application Connection

- Smorphi application was installed on our smartphones, from PlayStore.

- After installing, we scanned the room through the app, using BLE ( Bluetooth Low Energy ) technology.

- Once, Smorphi is detected in the list, we connect to it and launch the controller.

- Then, we were introduced with various buttons as a JoyStick, several shape transforming controls [ O, I, L, T, J, S, Z ], and 2 Pivot Turning buttons, which when held, pivoted the robot in clockwise and anti-clockwise directions.

Step#19: Sensor Configuration [ Sound Sensor ]

- A sound sensor was connected w/ a 4-pin cable to the Module 1, GPIO0 port.

- Then, it was coded through Arduino IDE.

- For the programming purpose, the USB A to C cable was connected from the Masterboard to our laptop.

- The laptop was previously set-up with the necessary drivers [ CP210x USB to UART Driver ], and necessary libraries in the IDE.

- After the connection, a sound sensor code was uploaded onto the laptop and then our robot was ready to perform the Sound Sensory Acts.

_ Disassembly _